Spatiotemporal Shoreline and Sea Level Dynamics along the Kenyan Coast (2000–2024): Patterns of Erosion, Accretion, and Spatial Autocorrelation

Mbugua Jamesa,b, Watene Georgea,,, Harison K. Kipkuleia,f  Ogega Obedd,e, Osuka Kennedy,a,g Obura Davidb,c

a Department of Geomatic Engineering and Geospatial Information Systems, Jomo Kenyatta University of Science and Technology, Kiambu, Kenya;

b Coastal Oceans Research & Development Indian Ocean, Mombasa, Kenya;

cIntergovernmental Science-Policy Platform on Biodiversity and Ecosystem Services (IPBES).

dInstitute for Climate Change and Adaptation, University of Nairobi, Nairobi, Kenya

eDepartment of Chemistry and Physics, Mount Saint Vincent University, Halifax, NS Canada

fCentre for Climate Resilience, University of Augsburg, Universitätsstraße 12, 86159, Augsburg, Germany

¶. g University of Liverpool School of Environmental Sciences Faculty of Science and Engineering

*Correspondence: Mbugua James (jamesmmbugua@gmail.com).

Keywords: Shoreline, sea level variability, GIS, Remote Sensing

Abstract

Shoreline management in the Western Indian Ocean (WIO) is largely informed by small-scale studies with coarse temporal resolution, potentially obscuring broader spatial patterns and interannual dynamics. This study examines spatiotemporal shoreline change along 400 km of the Kenyan coastline from 2000 to 2024 and evaluates the influence of sea-level anomalies (SLA), accounting for spatial dependence and heterogeneity. Annual Landsat-derived shoreline positions were analyzed using Digital Shoreline Analysis System (DSAS) and spatial statistical methods, including Moran’s I, Ordinary Least Squares (OLS), a Spatial Lag Model (SLM), and Geographically Weighted Regression (GWR). Results reveal pronounced spatial heterogeneity. The Kilifi and Kwale–Mombasa zones exhibit net accretionary trends (mean Weighted Linear Regression (WLR) rate ≈ 0.4 m yr⁻¹), whereas the Tana–Manda zone records persistent erosion (mean WLR = −0.6 m yr⁻¹). The national mean Net Shoreline Movement (NSM) of +10.7 m obscures local-scale variability. Spatial clustering is strong (Moran’s I = 0.799, p < 0.001), with 67% of transects classified as Low–Low stability clusters and 27% as High–High erosion or accretion hotspots. OLS indicates a weak association between shoreline change rates and SLA (R² = 0.006). Accounting for spatial dependence, the SLM shows no significant global relationship (p = 0.245), whereas GWR identifies substantial spatial non-stationarity. These findings demonstrate that large-scale remote sensing analyses must explicitly account for spatial heterogeneity and dependence to resolve coastal change drivers to inform shoreline management policy across scale.

1. Introduction

Shoreline dynamics, manifested through erosion and accretion, are key parameters in the study of coastal geomorphology and reflect the influence of both natural and anthropogenic processes (Baig et al. 2020). Human-induced factors, particularly the construction of grey infrastructure such as dams, reservoirs, seawalls, and ports, have been shown to disrupt natural sediment transport processes, resulting in sediment deficits and unstable shorelines (Mann et al. 2023, Hossain et al. 2025). Despite their inherent vulnerability, coastal regions are home to over 2.5 billion people, many of whom live in low-lying areas (Reimann et al. 2023, Zambrano-Medina et al. 2023, Depledge 2024). Population growth across coastal zones not only drives shoreline change (Mishra et al. 2020, Ankrah 2024) but also heightens exposure to climate-related hazards (Vousdoukas et al. 2022), underscoring the need for integrated shoreline management and climate-adaptive planning in rapidly urbanizing coastal areas such as Mombasa.

Globally, approximately 48% of the world’s sandy beaches are morphodynamically stable, while about 25% are undergoing net erosion at a mean shoreline retreat rate of ~0.5 m yr⁻¹ (Luijendijk et al. 2018). Retreating trends have also been reported in several parts of East Africa. For example, Njiiri et al. (2024) reported mean erosion rates of −3.57 m/year and −1.31 m/year on the South and North coasts of Kenya, respectively, while Alemayehu et al. (2014) found that 60% of the beaches in Watamu were eroding at a rate of 0.7m/year.

Understanding the spatial and temporal dynamics of shoreline change in this region is therefore essential for coastal planning, policymaking, and management. Although Digital Earth Africa (2020) provides a continental-scale baseline for shoreline change monitoring across Africa using consistent satellite-derived data, it does not explicitly account for positional uncertainty, and its continental scope limits its capacity to resolve fine-scale spatial variability in coastal dynamics.

In parallel, several site-specific studies Shaghude et al. (2003), Alemayehu et al. (2014), Ngowo et al.(2021), Aketch et al. (2022), Njiiri et al. (2024) and, Mbezi et al. (2024) are constrained by limited spatial coverage and coarse temporal resolution, limiting their capacity to capture interannual variability. As a result, spatial scale limitations may obscure the magnitude and spatial distribution of shoreline change, thereby constraining evidence-based coastal policy and management (Ankrah 2024).

To address these gaps, this study integrates the Automatic Satellite Derived Shoreline (Aedla et al. 2015, Hossain et al. 2021, Wang 2023) and  DSAS (Thieler et al. 2009), with scalable processing platforms Google Earth Engine (GEE) (Gorelick et al. 2017) to produce more spatially extensive and temporally resolved assessments across the Kenyan coastline. Annual shoreline positions measured along 30-m transects are used to resolve interannual variability.

Accordingly, the objectives of this study are threefold: (i) to characterize the magnitude and direction of shoreline change over the past 24 years (2000–2024); (ii) to quantify the influence of sea-level variability on observed shoreline dynamics; and (iii) to examine the spatial structure, autocorrelation, and heterogeneity of shoreline change patterns through global and local geostatistical modelling, to identify spatially varying drivers and dependencies governing coastal dynamics along the Kenyan coastline.

2. Methodology

2.1 Study area

This study covers approximately 400 km (about 75%) of Kenya’s coastline, extending from Manda Island in Lamu County to Vanga in Kwale County at the Kenya–Tanzania border. For analytical purposes, the study area was subdivided into three zones: the fringing reef shoreline of Kwale–Mombasa, the deltaic shoreline of Kilifi County encompassing the Sabaki and Tana rivers, and the ancient delta area of Tana–Manda (Figure 1).

The continental shelf along this coastline is narrow, steep  and varies considerably ranging from about 5 km in the south to 60 km in the north (Kamau et al. 2021). This variation influences local marine environments, with the broader northern shelf supporting extensive shallow-water habitats and productive fisheries (Jacobs et al. 2020, Kamau et al. 2021), while the narrower southern shelf is marked by steeper bathymetric gradients. The coastline supports diverse ecosystems, notably estuarine systems at the mouths of the River Tana and Sabaki, fringing coral reefs, seagrass beds, mangrove forests, and associated coastal forests. Recent red listing of coral reefs has categorized the entire stretch as vulnerable to collapse (Obura et al. 2022), underscoring the need for targeted shoreline management.

2.2 Data and study workflow

Remotely sensed and in situ data were obtained from open-access repositories and processed for the Kenyan coastline. First, optical satellite imagery was used to extract historical shoreline positions and compute shoreline change rates. Second, SLA data were used as a predictor variable in geostatistical analyses to examine the relationship between shoreline dynamics and sea level variability, while in situ tide gauge data were used to quantify uncertainty in shoreline position. Figure 2 provides a schematic summary of the methodological approach adopted in this study, linking the primary research objectives to the specific analytical procedures used for data collection, processing, and interpretation.

2.2.1 Analysis of historical (2000 - 2024) shoreline patterns

Shoreline positions from 2000 to 2024 were extracted from Landsat 7 Enhanced Thematic Mapper Plus (ETM+, operational from 1999) and Landsat 8 Operational Land Imager (OLI, operational from 2013) Level 2 Surface Reflectance (SR) imagery produced by the Earth Resources Observation and Science Center (EROS). The data were accessed from the U.S. Geological Survey (U.S.G.S 2021) Landsat archive via the Google Earth Engine (GEE) platform (Gorelick et al. 2017).

Landsat Collection 2 SR products were selected for their improved geolocation accuracy relative to earlier collections, with geometric root mean square error (RMSE) typically below one pixel, and their consistent radiometric calibration across the archive, making them suitable for multi-decadal change detection (U.S.G.S 2021). Landsat 8 OLI data were used for the 2021–2024 period to ensure sensor consistency. Sentinel-2 Multispectral Instrument (MSI) imagery, while offering superior spatial resolution (10 m), was not incorporated due to its limited temporal coverage prior to 2015, which would have precluded a consistent interannual time series spanning the full 2000–2024 study period.

GEE was adopted as the primary processing environment for this study, enabling scalable and reproducible automated shoreline delineation (Wang 2023). Shoreline delineation was conducted following a systematic procedure. First, Quality Assessment (QA) bands were used to mask cloud-contaminated and radiometrically saturated pixels. The QA_PIXEL band was used to identify and remove clouds, cloud shadows, and other low-quality pixels, while the QA_RADSAT band was used to exclude saturated pixels.

Surface reflectance values were obtained using the multiplicative scale factor and additive offset provided by the U.S. Geological Survey (2021). For each optical band, the true surface reflectance (ρ) was computed (eq 1).

ρ= M ×Qcal+ A	(1)

Where Qcal​ is the scaled reflectance value (digital number), M is the multiplicative scaling factor, and A is the additive offset.

Annual median composite images were generated from rescaled surface reflectance Landsat Collection 2 Level-2 data to reduce residual atmospheric noise, cloud contamination, and short-term hydrodynamic variability that affect single-date shoreline extraction (Hagenaars et al. 2018, Digital Earth 2020).

The Automated Water Extraction Index (AWEI) was employed  following Feyisa et al. (2014) to suppress misclassification errors arising from shadow, spectrally dark surfaces, and urban spectral interference. For Landsat 7 ETM+, the Green (Band 2), NIR (Band 4), SWIR1 (Band 5), and SWIR2 (Band 7) bands were used, while for Landsat 8 OLI, the corresponding Green (Band 3), NIR (Band 5), SWIR1 (Band 6), and SWIR2 (Band 7) bands were applied. The AWEI values were computed using Equation (2), and a scene-specific threshold derived from histogram analysis was subsequently applied to produce a binary water/non-water classification.

AWEI = 4* Green - SWIR2- 0.25 * NIR + 2.75 * SWIR1 	(2)

Water and land pixels were classified using an adaptive threshold derived from the Otsu (1979) algorithm, which determines the optimal threshold by minimizing intra‑class variance in the AWEI histogram. This implementation followed the approach of Donchyts et al. (2016)  operationalized within the GEE environment.

The initial binary land-water mask was subsequently refined using connected-component analysis and morphological filtering. Inland water bodies smaller than 1,000 m in diameter were removed from the land class, while islands smaller than 2,000 m were excluded from the water class. These size‑based filters reduced classification noise and minimized false detections along the coastal boundary.

Following refinement, vector polygons were generated from the binary raster using the reduceToVectors() function in GEE. Polygon boundaries were then simplified to polylines representing the land-water interface producing shoreline vectors through an automated raster‑to‑vector conversion process. Twenty-four vector shoreline positions (2000–2024) were delineated and archived as vector shapefiles for subsequent shoreline change rates analysis.

2.2.2 Estimating uncertainty in shoreline

Satellite-derived shoreline positions are subject to several sources of uncertainty, primarily related to image rectification, pixel resolution, seasonal wave conditions, tidal variability, and cloud contamination (Pardo-Pascual et al. 2018, Munasinghe and Warnasuriya 2023, Ankrah 2024, Zachopoulos et al. 2025)

This study employed automated shoreline delineation, thereby avoiding the subjective manual digitization errors inherent in visual interpretation. Nevertheless, tidal variability remains a recognized source of positional uncertainty; Boak and Turner (2005) demonstrate that shoreline position is strongly dependent on the tidal indicator used, and that referencing shorelines to a consistent vertical datum, such as mean high water, is essential for minimizing tidal-related positional variability across multi-temporal datasets. This tidal influence is particularly pronounced on low-gradient beaches, where modest water-level fluctuations can translate to substantial landward or seaward displacement of the mapped waterline (Fletcher et al. 2003, Castelle et al. 2021).

Subsequently, three sources of positional uncertainty were considered (Figure3): (i) spatial resolution error (Ures) estimated as half the pixel size of the Landsat image (Crowell et al. 1991, Thieler and Danforth 1994) (ii) shoreline displacement associated with tidal variability (Utide) calculated from this study using monthly mean tide gauge observations obtained from the Permanent Service for Mean Sea Level (PSMSL) database (Holgate et al. 2013) and (iii) image reregistration error derived from the Landsat geometric RMSE (Ugeo) available in the Landsat Metadata file (MTL) (2021). These independent components were combined in equation 3 using the root-sum-of-squares approach to estimate the total shoreline position uncertainty (Utotal).

Uₜₒₜₐₗ = Uᵣₑₛ2+Uₜᵢdₑ2+Ugeo2                  (3)

The combined uncertainty Uₜₒₜₐₗ was used as a weighting factor (equation 4) to estimate WLR change rates based on the approach by Baig et al. (2020) and Manno et al. (2017). Each shoreline was attributed with its positional uncertainty, where shorelines with lower positional uncertainty received greater weight (Thieler et al. 2009).

w=1Utotal2	                         		(4)

3. Data Analysis

3.1 Evaluation of shoreline position change rates

An offshore baseline was digitized approximately 500 m seaward of the derived shorelines. A total of 14,959 transects were then systematically cast orthogonally from the baseline at 30 m spacing along the approximately 400 km section of the Kenyan coastline. Each transect extended both landward and seaward to ensure intersection with the most landward and seaward shoreline positions (Figure4).

The transect intersection nodes between each transect and the shorelines were used to generate measurement points, which served as the basis for quantifying shoreline change. At each intersection point, positional measurements were recorded in meters relative to the baseline.

Shoreline change from 2000 to 2024 was quantified at each transect using the DSAS v5.0 (Thieler et al. 2009), selected for its widely adopted, robust framework for computing shoreline change from time-stamped shoreline vectors (Mishra et al. 2022). Annual satellite-derived shoreline positions generated in this study were attributed with acquisition year and positional uncertainty, and shoreline change was quantified using three established metrics: EPR, NSM, and WLR (Thieler et al. 2009, Baig et al. 2020, Mishra et al. 2022).

End Point Rate was computed (Eq. 5) as the rate of shoreline changes between the earliest (2000) and most recent (2024) shoreline positions:

EPR=Dlast-DfirstT(5)

where Dlastand Dfirstrepresent shoreline distances at the most recent and earliest epochs, respectively, and Tis the elapsed time interval (24 years).

Net Shoreline Movement was computed (Eq. 6) as the total horizontal displacement between the earliest and most recent shoreline positions:

NSM=Dlast-Dfirst(6)

Weighted Linear Regression was used to estimate shoreline change rates, incorporating shoreline positional uncertainty and all available shoreline observations. Positional uncertainty for each shoreline was quantified (Section 2.2, Eq. 3) as the root-sum-of-squares of rectification/registration error, image resolution error, and tidal variation error, and applied as weights in the regression model. For each transect, DSAS produced the WLR rate (m yr⁻¹), 90% confidence interval, standard error, and coefficient of determination. WLR was selected as the primary metric because it provides a more robust estimate of shoreline change.

Finally, spatial filtering to remove outliers was applied to the rate metric only using the 1.5 × interquartile range (IQR) criterion and filtered before aggregation, following the exploratory data analysis framework introduced by Tukey (1977) and widely adopted in contemporary data preprocessing practice.

3.2 Analysis of predictor variables: Kriging interpolation of sea-level anomaly data

Monthly mapped level 4 SLA data version vDT2024 was sourced from the Copernicus Climate Data Store (CDS). This dataset provides global mean estimates of SLA based on satellite altimetry measurements at 0.25° (~25 km)  spatial resolution (Lopez 2018). The data were obtained as a global-extent product and were spatially subset to the Kenyan coastline to match the defined study area. Subsequent temporal processing was performed using Climate Data Operators (CDO) software (Schulzweida et al. 2019), whereby mean SLA values were computed across the 2000-2024 time window (matching the WLR trend period) at each grid cell to derive a single grid dataset of time-averaged SLA representing  static, cross sectional sea level exposure. To spatially integrate the coarse-resolution SLA predictor with the 30 m transect framework, SLA data were resampled from the native 0.25° grid to the 30 m transect spacing using Ordinary Kriging (Cressie 1993) with Ordinary Kriging evaluated directly at each transect. Variogram fitting yielded: partial sill = 0.00066, full sill = 0.0006, range = 4.13°, nugget ≈ 0.00, confirming a spatially smooth SLA field at the measurement scale.

SLA data processing was accomplished in Jupyter Notebook (Kluyver et al. 2016), a web-based computing environment that supports Python-based data analysis and visualization workflows. The derived mean SLA data were spatially integrated with shoreline change rate metrics and used as predictors in the geostatistical analysis of shoreline changes.

3.3 Estimation of drivers of shoreline dynamics

The dynamic nature of shoreline change arises from multiple interacting coastal processes at the land–sea interface (Neumann et al. 2015, Mishra et al. 2022) together with meteorological forcing and human activities (Woodroffe et al. 2023). Rapid population growth in coastal zones is identified as a driver of shoreline pressure (Mishra et al. 2022), while sea level rise, largely linked to anthropogenic climate change, represents one of the most direct large-scale consequences influencing coastal stability (Nicholls and Cazenave 2010, Church et al. 2013, Neumann et al. 2015).

Spatial autocorrelation was evaluated using five geostatistical frameworks to examine spatial patterns and the relationships between shoreline change and sea level variability. First, a global OLS regression was applied to quantify the overall linear association between WLR shoreline rates and SLA. To assess whether shoreline change exhibited spatial dependence, Moran’s I statistic (Moran 1950, Chen 2023) was computed, and statistical significance was evaluated using a permutation test (999 permutations). Building on this global assessment, Local Indicators of Spatial Autocorrelation (LISA) (Anselin 1995) were calculated to identify spatial clusters and local outliers of shoreline change.

To assess whether spatial dependence was confounding the global OLS results, SLM was estimated as a diagnostic intermediate step between OLS and GWR (Anselin 1988). A k-nearest-neighbor (KNN, k = 30, row-standardized) weights matrix was used to define neighboring transects. GWR was employed to investigate spatial non-stationarity in the relationship between WLR and SLA (Brunsdon et al. 1996, Fotheringham et al. 2002). Unlike global regression approaches, GWR estimates location-specific coefficients, allowing the strength and direction of the shoreline–sea level relationship to vary continuously along the coastline and providing spatially explicit insight into process variability.

3.4 Site-based validation of shoreline change rates

Site-based validation was conducted through mobile mapping using Survey123 for ArcGIS Online (Esri 2025). Geolocated points were collected along field sites, with attribute data recorded, including photographs, beach width (m), cliff presence and condition, indicators of erosion or accretion, human interventions, and general field observations.

A total of eighty-two sites were surveyed between 15 and 25 July 2025 across the Kilifi and Kwale–Mombasa zones. Site observations were complemented by photographs documenting visible shoreline conditions, which serve both as verification material and as visual evidence of the changing coast. Parallel focus group discussions were conducted across representative communities in Kwale, Mombasa, and Kilifi to integrate local knowledge and historical accounts of shoreline change.

4. Results

4.1 National shoreline change rates (2000-2024)

Shoreline change metrics were computed using all available transects (n = 14,959) for the period 2000–2024 (Table 1). The national mean EPR was 0.5 m yr⁻¹ (median = 0.1 m yr⁻¹; SD = 4.9 m yr⁻¹). The mean WLR rate was 0.2 m yr⁻¹ (median = 0.1 m yr⁻¹; SD = 2.9 m yr⁻¹). The NSM showed a mean displacement of +10.7 m (median = 1.7 m; SD = 72.5 m). Shoreline positional uncertainty, accounting for pixel resolution, tidal variability, and image reregistration errors, was estimated at approximately ±21 m per shoreline epoch (Section 2.2.2, Eq. 3)

| Metric | n (transects) | Mean | Median | Std Dev |
| --- | --- | --- | --- | --- |
| EPR (m yr⁻¹) | 14,959 | 0.5 | 0.1 | 4.9 |
| WLR (m yr⁻¹) | 14,959 | 0.2 | 0.1 | 2.9 |
| NSM (m) | 14,959 | 10.7 | 1.7 | 72.5 |

Table 1. Summary statistics of national shoreline change metrics

The spatial distribution of NSM values along the coastline is shown in Figure 5. Transect-level displacement magnitudes are distributed along the three coastal zones (Kwale–Mombasa, Kilifi, and Tana–Manda), with variability evident across the full transect sequence. The accompanying transect-ordered plot (right panel) presents NSM values relative to Transect ID, illustrating the alongshore continuity of displacement patterns over the 2000–2024 period.

Zonal analysis of shoreline change rates (Table 2) revealed distinct spatial heterogeneity across the three coastal zones defined in Figure 1. Kilifi (n = 6,496) shows a mean EPR of 0.8 m yr⁻¹ and a mean WLR rate of 0.4 m yr⁻¹, with median values of 0.1 m yr⁻¹ for both metrics. Mean NSM in Kilifi is 17.9 m (SD = 85.1 m).

| Zone | n (transects) | Mean_EPR (m yr⁻¹) | Median_EPR (m yr⁻¹) | SD_EPR (m yr⁻¹) | Mean_WLR (m yr⁻¹) | Median_WLR (m yr⁻¹) | SD_WLR (m yr⁻¹) | Mean_NSM (m) | Median_NSM (m) | SD_NSM (m) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| KIlifi | 6,496 | 0.8 | 0.1 | 3.7 | 0.4 | 0.1 | 2.4 | 17.9 | 2.9 | 85.1 |
| Kwale_Mombasa | 4,688 | 0.8 | 0.1 | 5.4 | 0.4 | 0 | 2.7 | 14.7 | 1.7 | 61.2 |
| Manda_TanaRiver | 3,775 | -0.6 | -0.1 | 5.8 | -0.6 | -0.2 | 3.6 | -6.7 | -3 | 57.6 |

Table 2. Zonal summary statistics of shoreline change metrics across the three coastal zones of the Kenyan coastline (2000–2024).

The Kwale–Mombasa zone (n = 4,688) has a mean EPR of 0.8 m yr⁻¹ and a mean WLR of 0.4 m yr⁻¹, with median values of 0.1 m yr⁻¹ and 0.0 m yr⁻¹, respectively. Mean NSM in this zone is 14.7 m, with a standard deviation of 61.2 m. The Manda–Tana River zone (n = 3,775) exhibits negative shoreline change rates, with mean EPR and WLR values of −0.6 m yr⁻¹. Median EPR and WLR values of −0.1 m yr⁻¹ and −0.2 m yr⁻¹, respectively. Mean NSM is −6.7 m (SD = 57.6 m).

4.2 Spatial structure and predictors of shoreline change

To investigate spatial organization and drivers of shoreline change, Global Moran’s I, LISA, OLS, SLM, and GWR were applied.

4.2.1 Global Moran Index

Global Moran’s I revealed strong positive spatial autocorrelation in shoreline change rates (I = 0.799, permutation p = 0.001). This indicates statistically significant clustering of similar WLR values along the coast (Figure 6).

4.2.2 Local Indicators of Spatial Association

The LISA analysis classified 14,959 shoreline segments into distinct spatial autocorrelation categories. The Low-Low cluster category accounted for 67.28% of segments, while 26.57% were High-High. Spatial outliers comprised 2.96% Low-High and 3.19% High-Low. The spatial configuration of these clusters is shown in Figure 7.

4.2.3 Ordinary Least Squares

OLS regression indicated a negative linear association between SLA and WLR, with a constant coefficient of 0.916 and a slope coefficient of –8.517 for SLA (p < 0.001, F-statistic = 90.27), as shown in Figure 8. However, R2 was 0.006, signifying that SLA accounts for 0.6% of the observed variation in WLR. The adjusted R-squared remains unchanged due to the single predictor used in the model.

4.2.4 Spatial Lag Model

To account for spatial dependence, a SLM was employed as a diagnostic intermediate step between OLS and GWR. The SLM yielded a spatial autoregressive parameter of ρ = 0.953 (z = 334.05, p < 0.001), confirming strong positive spatial dependence in WLR rates, and a pseudo-R² = 0.813, reflecting a substantial improvement in explanatory power over OLS. Accounting for this spatial dependence removed SLA's direct effect on shoreline change rates (coefficient = −0.457, z = −1.16, p = 0.245, not significant), while the total effect incorporating spatial spillovers remained substantial. The total impact of SLA on shoreline change, incorporating spatial spillovers, was –9.776 m yr⁻¹ (direct impact = –0.457 m yr⁻¹; indirect spillover = –9.319 m yr⁻¹). These results justified applying GWR to examine whether the SLA–WLR relationship exhibits spatial non-stationarity.

4.2.5 Geographically Weighted Regression

The GWR model substantially improved model fit relative to OLS (adjusted R² = 0.882 vs. 0.006). Local SLA coefficient estimates ranged from –40,186.52 to +38,395.64 (mean = 17.31; median = 0.51; SD = 3,322.72), with the large gap between the mean and median indicating that a small number of extreme local estimates disproportionately influence the mean. The sign of the local SLA coefficient was nearly evenly split across transects (49.9% negative, 50.1% positive). Restricting to the subset of transects where the local coefficient was statistically significant at α = 0.05 (13.4% of transects, |t| ≥ 1.96) showed a similarly even split (49.6% negative, 50.4% positive), with the mean (–140.23) and median (415.02) among significant transects carrying opposite signs.

4.2.6 Site validation

The field survey revealed a shoreline characterized by widespread erosion hotspots, interspersed with areas of accretion and localized stability. Of 82 stations, 36 sites were classified as erosion hotspots, 27 as stable, 15 as accretion hotspots, and 2 as others (Figure 9). Accretion sites showed visible sand deposition, while stable sites typically retained intact natural vegetation cover.

Human interventions such as seawalls and beach walls were observed at 57 sites (interpreted in Section 5.2). Naturally vegetated areas were more stable.

Beach width ranged from approximately 22 meters to over 190 meters, with boxplot analysis showing marked differences between categories of erosion, accretion, and stability (Figure 10).

5. Discussion and Limitations

This study provides spatially extensive and temporally resolved assessments of shoreline dynamics along the Kenyan coast over 24 years. The findings reveal pronounced spatial heterogeneity in erosion and accretion, driven by a combination of geomorphic settings, sediment supply dynamics, and anthropogenic modification of coastal systems. These results demonstrate that aggregate national-scale metrics obscure the localized extremes most relevant to coastal management.

5.1 Spatial patterns and their management significance

The Kilifi and Kwale–Mombasa zones exhibit net accretionary trends (mean WLR ≈ 0.4–0.46 m yr⁻¹), while the Tana–Manda zone records persistent erosion (mean WLR = −0.6 m yr⁻¹). These contrasts reflect differences in geomorphic setting, sediment supply, and exposure to marine processes. National-scale averages indicate limited overall shoreline change. What matters for management, however, are the extreme erosion hotspots retreating well above the ±1.25 m yr⁻¹ detection threshold, and accretion sectors advancing sufficiently to generate land tenure disputes, both of which are rendered invisible by national averages. This underscores a fundamental governance challenge: coastal management policies built on aggregate assessments will systematically misallocate protection resources and overlook emerging risks.

5.2 Anthropogenic drivers and the human modification of sediment pathways

Ground validation confirmed that severe erosion hotspots coincide with significant anthropogenic modification of sediment pathways, consistent with broader evidence that human interventions intensify shoreline retreat (Anthony 2015). The Mukurumudzi River case reflects a broader pattern in which upstream dams act as a sediment sink (Katuva 2014). The downstream shoreline response at Vanga is consistent with the well-documented pattern of delta starvation following dam construction (Syvitski and Saito 2007). Environmental impact assessments incorporate mandatory downstream sediment budget modeling as a license condition, a requirement largely absent from current Kenyan coastal governance frameworks.

At Mambrui and Ngomeni, approximately 10,000 ha of mangrove clearance for salt production (Abuodha and Kairo 2001, Hamza and Esteves 2022), combined with impermeable dykes and seawalls, has altered tidal prism and nearshore circulation, consistent with documented linkages between mangrove loss and increased coastal hazard exposure along the Kenyan coast  (Hamza et al. 2022, Anthony et al. 2023). The reported loss of a naval structure and a hotel in Ngomeni illustrates the geomorphic impact of unregulated coastal land use. Site-based validation further showed that seawall protection is spatially uneven: structures near Beach Management Unit offices in Ngomeni provided localized protection, whereas many other seawalls coincided with increased erosion on adjacent, unprotected beaches, consistent with the broader pattern of engineered hardening displacing erosion alongshore. However, beaches buffered by intact natural vegetation in Watamu remained markedly more stable, and reef-backed sections of the Kwale-Mombasa coast demonstrated the protective value of ecosystem integrity. Malindi presents a divergent management challenge: progradation around the Vasco da Gama Pillar has generated emergent land, rendering existing jetty unusable for trawlers and triggering land ownership disputes. The absence of legal frameworks for contested emergent land requires policy.

5.3 Spatial structure, sea-level variability, and implications for monitoring

Shoreline change along the Kenyan coast is strongly clustered rather than randomly distributed (Moran’s I = 0.799, p < 0.001). LISA classification (67% Low–Low stability clusters; 27% High–High hotspots) reflects spatial continuity in erosion and accretion processes. This spatial dependence has direct methodological implications: monitoring programs built on isolated point observations (tide gauges, fixed beach profiles) will systematically underrepresent local extremes. An annually updated remote sensing framework of the type developed here is therefore needed for national coastal monitoring.

Regression analyses reveal geographically variable SLA influence on shoreline dynamics. The global OLS yielded a weak relationship (R² = 0.006), which improved markedly through spatial modeling. The SLM (pseudo-R² = 0.81) accounted for most spatial clustering, while GWR (adjusted R² = 0.882) revealed strong local heterogeneity in the SLA–shoreline relationship. Thus, sea-level forcing operates regionally; however, local geomorphic and anthropogenic conditions mediate its expression. This finding aligns with the broader literature emphasizing that site-specific factors govern coastal erosion responses (Pang et al. 2023) and carries an important message for regional sea-level rise adaptation planning: uniform retreat projections based on global SLA trends may misrepresent local risk. Translating these local coefficients into a zone- and hotspot-specific decision-support layer, rather than a single national retreat rate, would give county and national governments a more defensible basis for prioritizing setback lines, permitting decisions, and erosion-mitigation investment.

5.4 Comparison with existing literature and transferability

The zonal change rates reported here diverge from prior site-specific studies in instructive ways. Njiiri et al. (2024) reported mean erosion rates of −3.57 m yr⁻¹ and −1.31 m yr⁻¹ for the south and north coasts, respectively, markedly higher than the WLR values obtained here. This discrepancy reflects the tendency for short-record, single-date analyses to capture episodic retreat rather than secular trends, and the absence of positional uncertainty weighting in earlier studies. Alemayehu et al. (2014) recorded erosion across 60% of Watamu beaches at approximately 0.7 m yr⁻¹, directionally consistent with the present findings for Kilifi but differing in eroding transect proportion attributable to decadal variability and differences in shoreline indicator definition. The DEA (2020) continental assessment provides a useful regional benchmark but lacks explicit uncertainty weighting and fine-scale spatial resolution. This study corroborates Luijendijk et al. (2018) and Mentaschi et al.’s (2018) finding that shoreline change is highly heterogeneous within national coastlines. The methodological approach framework is transferable to other data-sparse coastlines in the Western Indian Ocean and sub-Saharan Africa, where long-term observational records are limited.

5.5 Limitations

The 30 m spatial resolution of Landsat imagery imposes a detection threshold of approximately ±1.25 m yr⁻¹, limiting confidence in low-magnitude trends. The root-sum-of-squares uncertainty propagation, while rigorous, does not capture systematic biases from inter-annual variability in tidal phase at image acquisition time. The exclusion of Landsat 9 represents a trade-off between sensor consistency and temporal completeness. Field validation covered 82 sites in Kilifi and Kwale–Mombasa but did not extend to the Tana–Manda zone, where the remotely sensed erosion signal is strongest. This gap is a priority for future fieldwork. The satellite signal in the Tana-Manda zone is nonetheless corroborated by independent evidence of mangrove loss, delta starvation, and structural damage (Anthony et al. 2023), collaborating the erosion trend identified here.

Interpretation of the GWR coefficient is constrained by the substantial resolution mismatch between the 25 km SLA product and the 30 m transect. Although Ordinary Kriging provided spatially continuous SLA estimates at transect locations, the effective spatial resolution of this variable remains approximately 25 km. Sub-grid SLA variability, which may be relevant at nearshore and coastal inlet scales, cannot be represented by this product and is not captured in the analysis. The SLA term should therefore be interpreted as a large-scale, low-frequency forcing signal. Future studies incorporating higher-resolution nearshore sea level products or tide-gauge observations would improve the spatial specificity of this predictor.

6.0 Conclusion

The spatial framework developed in this study presents a scalable approach for coastal monitoring across the WIO. The identification of erosion hotspots provides an opportunity for prioritizing coastal protection and integration of Environmental Impact Assessment (EIA) requirements for upstream infrastructure, and anchoring nature-based adaptation strategies within Kenya's Integrated Coastal Zone Management (ICZM) framework. Under Kenya's devolved governance system (County government Act 2012), the spatial delineation of coastal hazards including erosion and accretion zones is directly actionable by county governments developing County Integrated Development Plans (CIDP) and blue economy strategies. The workflow presented here is fully reproducible using open-access data and platforms, positioning it as a replicable model for data-sparse WIO coastlines where equivalent national assessments remain absent.

Sea-level anomalies exert a spatially variable influence on shoreline dynamics. The SLM shows that, once strong spatial dependence (ρ = 0.953) is modeled, SLA does not retain a statistically significant global effect (total impact including spillovers = –9.56 m yr⁻¹, p = 0.253), while GWR reveals a significant, spatially localized SLA effect that is masked at the global scale by geomorphic and anthropogenic conditions. Uniform sea-level rise projections risk misrepresenting local hazard exposure; spatially explicit assessments are essential for equitable climate adaptation planning.

Spatial autocorrelation analysis confirms that shoreline change is strongly clustered (Moran's I = 0.799, p < 0.001), with 67% of transects in Low-Low stability clusters and 27% in High-High erosion or accretion hotspots. This structure has direct implications for monitoring design: point-based observation networks systematically underrepresent local extremes, and an annually updated remote sensing framework of the type demonstrated here is required for effective national coastal monitoring.

This study presents a 24-year, spatially explicit assessment of shoreline dynamics along approximately 400 km of the Kenyan coastline, integrating satellite-derived shoreline positions, uncertainty-weighted change rate analysis, and a progressive suite of spatial statistical models. The results reveal pronounced spatial heterogeneity: net accretion in the Kilifi and Kwale-Mombasa zones and persistent erosion in the Tana-Manda zone. National-scale averages systematically obscure the localized extremes most relevant to coastal management.

CRediT authorship contribution statement

Funding

Declaration of Competing Interest

Acknowledgments

References

Abuodha, P., et al. (2001). "Human-induced stresses on mangrove swamps along the Kenyan coast." Hydrobiologia 458(1): 255–265.

Aedla, R., et al. (2015). "Automatic shoreline detection and change detection analysis of Netravati-Gurpur rivermouth using histogram equalization and adaptive thresholding techniques." Aquatic Procedia 4: 563–570.

Aketch, D. N., et al. (2022). "Monitoring evolution of coastline along Mto Tamamba Delta in Kenya north coast using satellite images." Journal of Sensors 2022: 1–12.

Alemayehu, F., et al. (2014). "Assessment of shoreline changes in the period 1969-2010 in Watamu area, Kenya."

Ankrah, J. (2024). "Shoreline change and coastal erosion: an analysis of long-term change and adaptation strategies in coastal Ghana." Geo-Marine Letters 44(3): 12.

Anselin, L. (1988). Spatial Econometrics: Methods and Models, Kluwer Academic Publishers.

Anselin, L. (1995). "Local indicators of spatial association—LISA." Geographical Analysis 27(2): 93–115.

Anthony, E. J. (2015). "Wave influence in the construction, shaping and destruction of river deltas: A review." Marine Geology 361: 53–78.

Anthony, E. J., et al. (2023). "Evaluating the combined impacts of hydrological change, coastal dynamics and human activity on mangrove cover and health in the Tana River delta, Kenya." Estuarine, Coastal and Shelf Science 285: 108270.

Baig, M. R. I., et al. (2020). "Analysis of shoreline changes in Vishakhapatnam coastal tract of Andhra Pradesh, India: an application of digital shoreline analysis system (DSAS)." Annals of GIS 26(4): 361–376.

Brunsdon, C., et al. (1996). "Geographically weighted regression: A method for exploring spatial nonstationarity." Geographical Analysis 28(4): 281–298.

Castelle, B., et al. (2021). "Satellite-derived shoreline detection at a high-energy meso-macrotidal beach." Geomorphology 383: 107707.

Chen, Y. (2023). "Spatial autocorrelation equation based on Moran's index." Scientific Reports 13: 19296.

Church, J. A., et al. (2013). "Sea-level rise by 2100." Science 342(6165): 1445.

Cressie, N. A. C. (1993). Statistics for Spatial Data. New York, John Wiley & Sons.

Crowell, M., et al. (1991). "Historical Shoreline Change: Error Analysis and Mapping Accuracy." Journal of Coastal Research 7(3): 839–852.

Depledge, M. H. (2024). "Re-thinking human interactions with the oceans." Royal Society Open Science 11: 240808.

Digital Earth, A. (2020). Digital Earth Africa Coastlines.

Donchyts, G., et al. (2016). "A 30 m resolution surface water mask including estimation of positional and thematic differences using Landsat 8, SRTM and OpenStreetMap." Remote Sensing 8(5): 386.

Elizabeth, H. B., et al. (2005). "Shoreline Definition and Detection: A Review." Journal of Coastal Research 2005(214): 688–703.

Esri (2025). ArcGIS Survey123. Redlands, California, Environmental Systems Research Institute.

Feyisa, G. L., et al. (2014). "Automated Water Extraction Index: A new technique for surface water mapping using Landsat imagery." Remote Sensing of Environment 140: 23–35.

Fletcher, C. H., et al. (2003). "Mapping Shoreline Change Using Digital Orthophotogrammetry on Maui, Hawaii." Journal of Coastal Research Special Issue No. 38: 106–124.

Fotheringham, A. S., et al. (2002). Geographically Weighted Regression: The Analysis of Spatially Varying Relationships. Chichester, UK, John Wiley & Sons.

Gorelick, N., et al. (2017). "Google Earth Engine: Planetary-scale geospatial analysis for everyone." Remote Sensing of Environment 202: 18–27.

Hagenaars, G., et al. (2018). "On the accuracy of automated shoreline detection derived from satellite imagery." Coastal Engineering 133: 113–125.

Hamza, A. J., et al. (2022). "Changes in Mangrove Cover and Exposure to Coastal Hazards in Kenya." Land 11(10): 1714.

Holgate, S. J., et al. (2013). "New Data Systems and Products at the Permanent Service for Mean Sea Level." Journal of Coastal Research 29(3): 493–504.

Hossain, M. S., et al. (2025). "Impacts of natural and anthropogenic changes on morphology and shoreline dynamics along the southeast coast of Bangladesh." Arabian Journal of Geosciences 18: 20.

Hossain, M. S., et al. (2021). "Automatic shoreline extraction and change detection: A study on the southeast coast of Bangladesh." Marine Geology 441: 106628.

Jacobs, Z. L., et al. (2020). "Shelf-break upwelling and productivity over the North Kenya Banks." Journal of Geophysical Research: Oceans 125: e2019JC015519.

Kamau, J. N., et al. (2021). "Managing emerging fisheries of the North Kenya Banks in the context of environmental change." Ocean & Coastal Management 209: 105671.

Katuva, J. M. (2014). Water Allocation Assessment: A Study of Hydrological Simulation on Mukurumudzi River Basin, University of Nairobi.

Kluyver, T., et al. (2016). Jupyter Notebooks: a publishing format for reproducible computational workflows. Positioning and Power in Academic Publishing: Players, Agents and Agendas. F. Loizides and B. Scmidt. Amsterdam, IOS Press: 87–90.

Lopez, A. (2018). Sea level daily gridded data from satellite observations for the global ocean from 1993 to present, ECMWF.

Luijendijk, A., et al. (2018). "The state of the world's beaches." Scientific Reports 8: 6641.

Mann, T., et al. (2023). "Multi-decadal shoreline changes in Eastern Ghana—natural dynamics versus human interventions." Geo-Marine Letters 43(4): 17.

Manno, G., et al. (2017). "Uncertainties in shoreline position analysis." Ocean Science 13: 661–671.

Mbezi, J., et al. (2024). "Exploring shoreline changes and their implications in coastal communities." Regional Studies in Marine Science 75: 103566.

Mentaschi, L., et al. (2018). "Global long-term observations of coastal erosion and accretion." Scientific Reports 8: 12876.

Mishra, M., et al. (2022). "Analyzing shoreline dynamicity along the southern Odisha coast of India." Geocarto International 37(14): 3991–4027.

Mishra, M., et al. (2020). "The development and research trend of using DSAS tool for shoreline change analysis: A scientometric analysis." Journal of Urban and Environmental Engineering 14(1): 69–77.

Moran, P. A. P. (1950). "Notes on continuous stochastic phenomena." Biometrika 37: 17.

Munasinghe, T. N., et al. (2023). "Spatiotemporal behaviour of shorelines due to natural and anthropogenic influences on the Dehiwala-Mt. Lavinia Beach, Sri Lanka: Insights towards effective application of satellite-derived data and geospatial techniques." Ocean & Coastal Management 242: 106734.

Neumann, B., et al. (2015). "Future coastal population growth and exposure." PLOS ONE 10: e0118571.

Ngowo, R. G., et al. (2021). "Quantifying 28-year (1991–2019) shoreline change trends along the Mnazi Bay – Ruvuma Estuary Marine Park, Tanzania." Remote Sensing Applications: Society and Environment 23: 100607.

Nicholls, R. J., et al. (2010). "Sea-level rise and its impact on coastal zones." Science 328: 1517–1520.

Njiiri, W., et al. (2024). "Determination of Shoreline Variability for Adaptation of Maritime Built Heritage to Climate Change: A Case of Southern Kenya Coast." The Historic Environment: Policy & Practice 15: 594 – 610.

Obura, D., et al. (2022). "Vulnerability to collapse of coral reef ecosystems in the Western Indian Ocean." Nature Sustainability 5(2): 104–113.

Otsu, N. (1979). "A threshold selection method from gray-level histograms." IEEE Transactions on Systems, Man, and Cybernetics 9: 62–66.

Pang, T., et al. (2023). "Coastal erosion and climate change: A review." Ambio 52: 2034–2052.

Pardo-Pascual, J. E., et al. (2018). "Assessing the accuracy of automatically extracted shorelines on microtidal beaches from Landsat 7, Landsat 8 and Sentinel-2 imagery." Remote Sensing 10.

Reimann, L., et al. (2023). "Population development as a driver of coastal risk: Current trends and future pathways." Cambridge Prisms: Coastal Futures 1: e14.

Schulzweida, U., et al. (2019). Climate Data Operator (Version 1.9.6), Zenodo.

Shaghude, Y. W., et al. (2003). "Assessment of shoreline changes in the western side of Zanzibar Channel." International Journal of Remote Sensing 24: 4953–4967.

Syvitski, J. P. M., et al. (2007). "Morphodynamics of deltas under the influence of humans." Global and Planetary Change 57: 261–282.

Thieler, E. R., et al. (1994). "Historical shoreline mapping (I)." Journal of Coastal Research 10(3): 549–563.

Thieler, E. R., et al. (2009). Digital Shoreline Analysis System (DSAS) Version 4.0, USGS.

Tukey, J. W. (1977). Exploratory Data Analysis. Reading, Massachusetts, Addison-Wesley.

U.S.G.S (2021). Landsat collection 2. Fact Sheet. Reston, VA.

Vousdoukas, M. I., et al. (2022). "African heritage sites threatened as sea-level rise accelerates." Nature Climate Change 12: 256–262.

Wang, W. (2023). "Automatic extraction of coastline based on Google Earth Engine." International Journal of Computer Science and Information Technology 1: 102–111.

Woodroffe, C. D., et al. (2023). Coastal systems: The dynamic interface between land and sea. Research directions, challenges and achievements of modern geography. J. Bański and M. Meadows, Springer: 207–229.

Zachopoulos, K., et al. (2025). "A harmonized framework to assess coastal erosion blending copernicus marine data products and satellite imagery along Greek and Italian shorelines." Frontiers in Environmental Science Volume 13 - 2025.

Zambrano-Medina, Y. G., et al. (2023). "Assessment and forecast of shoreline change using geospatial techniques." Land 12: 782.